# PIPELINED RISC-V REFRENCE

# Architecture overview

5-stage in-order pipeline: IF -> ID -> EX -> MEM -> WB

Branch evaluation: resolved in ID stage (early branch), static not-taken prediction.

Harvard memory: separate instruction memory and data memory.

Two-level decode: main control unit -> ALU decoder.

# Stage registers

All stage registers: synchronous reset, flush (zero control lines), and hold (enable) inputs.

| IF/ID | ID/EX | EX/MEM | MEM/WB | Notes |
| --- | --- | --- | --- | --- |
| Instruction | RS1 (value) | RS2 (value) | WBdata | WBdata = MEMtoREG ? MEMdata : ALUrslt |
| PC value | RS2 (value) | ALUresult | RD |  |
|  | Imm value | RD | REGwr |  |
|  | Funct3 | REGwr |  |  |
|  | Funct7 | MEMwr |  |  |
|  | RD | MEMrd |  |  |
|  | REGwr | MEMtoREG |  |  |
|  | MEMwr / MEMrd |  |  |  |
|  | MEMtoREG |  |  |  |
|  | ALUop / ALUsrc |  |  |  |
|  | RS1addr / RS2addr |  |  |  |

# Control tables

Decoder logic is identical to the single-cycle design. The main decoder lives in the ID stage. Outputs either feed directly to in-stage units (ImmGen, branch blocks, bubble insertion) or are registered into ID/EX for use in later stages.

## Main decoder (opcode -> control signals)

| Instruction | Opcode | ALUop | ALUsrc | IMMsrc | REGwr | MEMwr | MEMrd | MEMtoREG | Bop | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-type | 0110011 | 00 | 0 | X | 1 | 0 | 0 | 0 | 00 | Propagates to ID/EX: REGwr, MEMwr, MEMrd, MEMtoREG, ALUop, ALUsrc |
| I-type (ALU) | 0010011 | 01 | 1 | 01 | 1 | 0 | 0 | 0 | 00 | IMMsrc -> ImmGen (ID stage) |
| LW | 0000011 | 10 | 1 | 01 | 1 | 0 | 1 | 1 | 00 | MEMrd=1 is key load identifier for hazard detection |
| SW | 0100011 | 10 | 1 | 11 | 0 | 1 | 0 | 0 | 00 | IMMsrc=11 -> S-immediate |
| BEQ | 1100011 | 11 | 0 | 10 | 0 | 0 | 0 | 0 | 01 | Bop -> branch eval block + bubble insertion; IMMsrc -> ImmGen -> BTA |
| BNE | 1100011 | 11 | 0 | 10 | 0 | 0 | 0 | 0 | 10 | Differs from BEQ only in Bop encoding |

## ALU decoder (ALUop + funct -> ALUctrl)

ALU encoding: ADD=000, SUB=001, AND=010, OR=011, XOR=100, SLT=101, SLL=110, SRL=111

| Type | ALUop | Funct7[5] | Funct3 | ALUctrl | Notes |
| --- | --- | --- | --- | --- | --- |
| R-type | 00 | 0 | 000 | ADD (000) |  |
| R-type | 00 | 1 | 000 | SUB (001) | funct7[5]=1 distinguishes SUB from ADD |
| R-type | 00 | 0 | 111 | AND (010) |  |
| R-type | 00 | 0 | 110 | OR (011) |  |
| R-type | 00 | 0 | 100 | XOR (100) |  |
| R-type | 00 | 0 | 010 | SLT (101) |  |
| R-type | 00 | 0 | 001 | SLL (110) |  |
| R-type | 00 | 0 | 101 | SRL (111) |  |
| I-type ALU | 01 | X | funct3 | same as R-type | ALUsrc=1; funct7[5] ignored (except SRAI, deferred) |
| LW / SW | 10 | X | 010 | ADD (000) | Address = rs1 + imm |
| BEQ | 11 | X | 000 | SUB (001) | Zero flag from subtraction drives branch eval block |
| BNE | 11 | X | 001 | SUB (001) | Same operation; Bop distinguishes BEQ vs BNE outcome |

## IMMsrc encoding

| IMMsrc | Type | Immediate field | Used by |
| --- | --- | --- | --- |
| 00 | I-immediate | inst[31:20] | ADDI and all I-type ALU, LW |
| 10 | B-immediate | {inst[31],inst[7],inst[30:25],inst[11:8],1'b0} | BEQ, BNE - fed to branch target adder |
| 11 | S-immediate | {inst[31:25], inst[11:7]} | SW |

# Branch handling modules

## Branch target adder  [DONE]

Computes the branch target address: PC + sign-extended immediate.

| Signal / Field | Detail |
| --- | --- |
| INPUTS / OUTPUTS |
| In_PC | Source: IF/ID.PC |
| In_IMM | Source: ImmGen output (sign-extended B-immediate) |
| Branch_target | Output -> PC mux (PCsrc) |
| LOGIC / BEHAVIOUR |
| Function | Branch_target = In_PC + In_IMM |

## COMP (equality comparator)  [DONE]

Compares two 32-bit values. Embedded within the branch evaluation block.

| Signal / Field | Detail |
| --- | --- |
| INPUTS / OUTPUTS |
| COMP_A | Source: branch forwarding mux A output |
| COMP_B | Source: branch forwarding mux B output |
| NEQ_flag | Output: 0 if A == B, 1 if A != B |
| LOGIC / BEHAVIOUR |
| NEQ_flag | NEQ_flag = (COMP_A != COMP_B) ? 1 : 0 |

## Branch evaluation block  [DONE]

Evaluates BEQ / BNE outcome using COMP result and Bop encoding. Drives PCsrc.

| Signal / Field | Detail |
| --- | --- |
| INPUTS / OUTPUTS |
| Bop[1:0] | Source: control decoder. 00=no branch, 01=BEQ, 10=BNE |
| COMP_A | Source: branch forwarding block output A |
| COMP_B | Source: branch forwarding block output B |
| BRANCH | Output -> PCsrc (selects PC+4 or branch target) |
| LOGIC / BEHAVIOUR |
| BEQ path | BRANCH = BEQ & ~NEQ_flag (branch if A == B) |
| BNE path | BRANCH = BNE & NEQ_flag (branch if A != B) |
| Combined | BRANCH = (Bop==01 & ~NEQ_flag) ¦ (Bop==10 & NEQ_flag) |

## Branch forwarding and detection  [DONE]

Selects correct operands for COMP by forwarding from EX/MEM or MEM/WB when a RAW hazard exists on a branch operand. Operates on the IF-stage branch instruction (pre-ID).

| Signal / Field | Detail |
| --- | --- |
| INPUTS / OUTPUTS |
| IF.Bop[1:0] | Branch type indicator from instruction decode |
| IF/ID.RS1 | Branch source register 1 address |
| IF/ID.RS2 | Branch source register 2 address |
| EX/MEM.RD | Destination register of instruction in EX/MEM |
| EX/MEM.REGwr | Register write enable of instruction in EX/MEM |
| EX/MEM.MEMrd | Memory read flag (load) of instruction in EX/MEM |
| MEM/WB.RD | Destination register of instruction in MEM/WB |
| MEM/WB.REGwr | Register write enable of instruction in MEM/WB |
| IF.REG_A | Register file output A (unforwarded) |
| IF.REG_B | Register file output B (unforwarded) |
| EX/MEM.ALUrslt | ALU result from EX/MEM stage |
| MEM/WB.WBdata | Write-back data from MEM/WB stage |
| COMP_A | Output -> COMP input A |
| COMP_B | Output -> COMP input B |
| LOGIC / BEHAVIOUR |
| Priority 1 | EX/MEM.REGwr & ~EX/MEM.MEMrd & (RS1==EX/MEM.RD) -> COMP_A=ALUrslt, COMP_B=REG_B |
| Priority 2 | EX/MEM.REGwr & ~EX/MEM.MEMrd & (RS2==EX/MEM.RD) -> COMP_A=REG_A, COMP_B=ALUrslt |
| Priority 3 | MEM/WB.REGwr & (RS1==MEM/WB.RD) -> COMP_A=WBdata, COMP_B=REG_B |
| Priority 4 | MEM/WB.REGwr & (RS2==MEM/WB.RD) -> COMP_A=REG_A, COMP_B=WBdata |
| Default | No hazard -> COMP_A=REG_A, COMP_B=REG_B |
| Note | EX/MEM forwarding excluded for loads (MEMrd=1). Load-branch case handled by bubble insertion (2 stalls required). |

# Hazard detection modules

## Bubble insertion unit  [DONE]

Detects load-use and branch hazards requiring pipeline stalls. On detection: holds PC, holds IF/ID register, zeros all control lines in ID/EX (inserts bubble). Purely combinational - no state machine.

| Signal / Field | Detail |
| --- | --- |
| INPUTS / OUTPUTS |
| IF.Bop[1:0] | INPUT - Branch type from IF-stage instruction decode (01=BEQ, 10=BNE, 00=none) |
| IF/ID.RS1[4:0] | INPUT - Source register 1 address of instruction in ID |
| IF/ID.RS2[4:0] | INPUT - Source register 2 address of instruction in ID |
| ID/EX.RD[4:0] | INPUT - Destination register address of instruction in EX |
| ID/EX.REGwr | INPUT - Register write enable of instruction in EX |
| ID/EX.MEMrd | INPUT - Memory read (load) flag of instruction in EX |
| EX/MEM.RD[4:0] | INPUT - Destination register address of instruction in MEM |
| EX/MEM.REGwr | INPUT - Register write enable of instruction in MEM |
| EX/MEM.MEMrd | INPUT - Memory read (load) flag of instruction in MEM |
| stall | OUTPUT - Asserted when any condition fires. Drives: PC.EN=0, IF/ID.EN=0, ID/EX control lines=0 |
| LOGIC / BEHAVIOUR |
| Condition 1 | (ID/EX.MEMrd ¦ IF.Bop) & ID/EX.REGwr & (IF/ID.RS1==ID/EX.RD ¦ IF/ID.RS2==ID/EX.RD) |
|  | Load-use OR branch with dependency on instruction currently in EX. Inserts first bubble. |
| Condition 2 | EX/MEM.MEMrd & EX/MEM.REGwr & (IF/ID.RS1==EX/MEM.RD ¦ IF/ID.RS2==EX/MEM.RD) & IF.Bop |
|  | Load-branch second stall: load is now in MEM, branch still in ID. Mutually exclusive with Condition 1 by construction - ID/EX holds a bubble when this fires, so ID/EX.REGwr=0 prevents Condition 1 re-firing. |
| Output | stall = Condition1 ¦ Condition2 |
| Stall actions | PC.EN=0 (hold PC) ¦ IF/ID.EN=0 (hold instruction) ¦ ID/EX control lines=0 (zero all, insert bubble) |

## ALU forwarding unit  [DONE]

Detects EX-stage RAW data hazards and selects the correct forwarded value for ALU inputs A and B. A and B are handled by independent priority chains. Implemented as combinational mux logic - no sequential cost.

| Signal / Field | Detail |
| --- | --- |
| INPUTS / OUTPUTS |
| ID/EX.RS1addr[4:0] | INPUT - Source register 1 address of instruction in EX |
| ID/EX.RS2addr[4:0] | INPUT - Source register 2 address of instruction in EX |
| EX/MEM.RD[4:0] | INPUT - Destination register address of instruction in MEM |
| EX/MEM.REGwr | INPUT - Register write enable of instruction in MEM |
| MEM/WB.RD[4:0] | INPUT - Destination register address of instruction in WB |
| MEM/WB.REGwr | INPUT - Register write enable of instruction in WB |
| ID/EX.RS1[31:0] | INPUT - RS1 data value from register file (unforwarded) |
| ID/EX.RS2[31:0] | INPUT - RS2 data value from register file (unforwarded) |
| EX/MEM.ALUrslt[31:0] | INPUT - ALU result from MEM stage (forwarding source) |
| MEM/WB.WBdata[31:0] | INPUT - Write-back data from WB stage (forwarding source) |
| ALU.A[31:0] | OUTPUT - Forwarded operand to ALU input A |
| ALU.B[31:0] | OUTPUT - Forwarded operand to ALU input B (note: ALUsrc mux selects between ALU.B and immediate downstream) |
| LOGIC / BEHAVIOUR |
| --- ALU.A chain --- | Independent priority chain for operand A |
| A: Fwd EX/MEM | EX/MEM.REGwr & (ID/EX.RS1addr == EX/MEM.RD) -> ALU.A = EX/MEM.ALUrslt |
| A: Fwd MEM/WB | MEM/WB.REGwr & (ID/EX.RS1addr == MEM/WB.RD) -> ALU.A = MEM/WB.WBdata |
| A: Default | No hazard -> ALU.A = ID/EX.RS1 |
| --- ALU.B chain --- | Independent priority chain for operand B |
| B: Fwd EX/MEM | EX/MEM.REGwr & (ID/EX.RS2addr == EX/MEM.RD) -> ALU.B = EX/MEM.ALUrslt |
| B: Fwd MEM/WB | MEM/WB.REGwr & (ID/EX.RS2addr == MEM/WB.RD) -> ALU.B = MEM/WB.WBdata |
| B: Default | No hazard -> ALU.B = ID/EX.RS2 |
| Priority note | EX/MEM takes priority over MEM/WB within each chain. A and B resolve independently. |

# Flush logic

Separate from hazard/bubble insertion. Driven directly by branch evaluation result.

| Condition | Action |
| --- | --- |
| BRANCH = 1 (branch taken) | IF_flush = 1 -> zero IF/ID control lines (discard fetched instruction) |
| BRANCH = 0 (not taken) | No flush - static not-taken prediction correct, pipeline continues normally |

- end of specification -