Single-Cycle RISC-V Reference

1. Instruction Formats

| Type | [31:25] | [24:20] | [19:15] | [14:12] | [11:7] | [6:0] |
| --- | --- | --- | --- | --- | --- | --- |
| R-type | funct7 | rs2 | rs1 | funct3 | rd | opcode |
| I-type | imm[11:0] | -- | rs1 | funct3 | rd | opcode |
| S-type | imm[11:5] | rs2 | rs1 | funct3 | imm[4:0] | opcode |
| SB-type | imm[12,10:5] | rs2 | rs1 | funct3 | imm[4:1,11] | opcode |
| U-type | imm[31:12] | -- | -- | -- | rd | opcode |
| UJ-type | imm[20,10:1,11,19:12] | -- | -- | -- | rd | opcode |

2. Control Signal Truth Table

| Instruction | Opcode | Funct7 | Funct3 | ALUctrl | ALUsrc | REGwr | MEMwr | MEMrd | MEMtoREG | PCsrc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD | 0110011 | 0000000 | 000 | 000 | 0 | 1 | 0 | 0 | 0 | 0 |
| SUB | 0110011 | 0100000 | 000 | 001 | 0 | 1 | 0 | 0 | 0 | 0 |
| AND | 0110011 | 0000000 | 111 | 010 | 0 | 1 | 0 | 0 | 0 | 0 |
| OR | 0110011 | 0000000 | 110 | 011 | 0 | 1 | 0 | 0 | 0 | 0 |
| XOR | 0110011 | 0000000 | 100 | 100 | 0 | 1 | 0 | 0 | 0 | 0 |
| SLT | 0110011 | 0000000 | 010 | 101 | 0 | 1 | 0 | 0 | 0 | 0 |
| SLL | 0110011 | 0000000 | 001 | 110 | 0 | 1 | 0 | 0 | 0 | 0 |
| SRL | 0110011 | 0000000 | 101 | 111 | 0 | 1 | 0 | 0 | 0 | 0 |
| ADDI | 0010011 | NA | 000 | 000 | 1 | 1 | 0 | 0 | 0 | 0 |
| ANDI | 0010011 | NA | 111 | 010 | 1 | 1 | 0 | 0 | 0 | 0 |
| ORI | 0010011 | NA | 110 | 011 | 1 | 1 | 0 | 0 | 0 | 0 |
| XORI | 0010011 | NA | 100 | 100 | 1 | 1 | 0 | 0 | 0 | 0 |
| SLTI | 0010011 | NA | 010 | 101 | 1 | 1 | 0 | 0 | 0 | 0 |
| SLLI | 0010011 | NA | 001 | 110 | 1 | 1 | 0 | 0 | 0 | 0 |
| SRLI | 0010011 | NA | 101 | 111 | 1 | 1 | 0 | 0 | 0 | 0 |
| LW | 0000011 | NA | 010 | 000 | 1 | 1 | 0 | 1 | 1 | 0 |
| SW | 0100011 | NA | 010 | 000 | 1 | 0 | 1 | 0 | 0 | 0 |
| BEQ | 1100011 | NA | 000 | 001 | 0 | 0 | 0 | 0 | 0 | 1* |
| BNE | 1100011 | NA | 001 | 001 | 0 | 0 | 0 | 0 | 0 | 1* |

* BEQ: PCsrc=1 when Zero=1  |  BNE: PCsrc=1 when Zero=0

3. ALU Decoder (2-Level Control)

| Type | ALUop | Funct7[5] | Funct3 | ALU Operation | Notes |
| --- | --- | --- | --- | --- | --- |
| R-type | 00 | 0 | 000 | ADD (000) |  |
| R-type | 00 | 1 | 000 | SUB (001) |  |
| R-type | 00 | 0 | 111 | AND (010) |  |
| R-type | 00 | 0 | 110 | OR (011) |  |
| R-type | 00 | 0 | 100 | XOR (100) |  |
| R-type | 00 | 0 | 010 | SLT (101) |  |
| R-type | 00 | 0 | 001 | SLL (110) |  |
| R-type | 00 | 0 | 101 | SRL (111) |  |
| I-type | 01 | X | funct3 | Same as R-type per funct3 | ALUsrc=1, use imm |
| LW/SW | 10 | X | 010 | ADD (000) | Address = rs1 + imm |
| BEQ | 11 | X | 000 | SUB (001) | Zero flag -> PCsrc |
| BNE | 11 | X | 001 | SUB (001) | ~Zero flag -> PCsrc |

4. Main Decoder (Opcode to Control Signals)

| Opcode | Type | ALUop | ALUsrc | IMMsrc | REGwr | MEMwr | MEMrd | MEMtoREG | Bop |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0110011 | R-type | 00 | 0 | X (none) | 1 | 0 | 0 | 0 | 00 |
| 0010011 | I-type (ALU) | 01 | 1 | 00 (I) | 1 | 0 | 0 | 0 | 00 |
| 0000011 | I-type (Load) | 10 | 1 | 00 (I) | 1 | 0 | 1 | 1 | 00 |
| 0100011 | S-type | 10 | 1 | 11 (S) | 0 | 1 | 0 | 0 | 00 |
| 1100011 | B-type | 11 | 0 | 10 (B) | 0 | 0 | 0 | 0 | 10/01 |

IMMsrc: 00=I-immediate  |  10=B-immediate  |  11=S-immediate  |  Bop: 10=BEQ  |  01=BNE

5. Instruction Descriptions

| Group | Instruction | Format | Description / Dataflow |
| --- | --- | --- | --- |
| R-type | ADD | R | rs1 + rs2 -> rd ¦ RegFile(rs1,rs2) -> ALU(ADD) -> RegFile(rd) |
|  | SUB | R | rs1 - rs2 -> rd ¦ ALU(SUB) -> RegFile(rd) |
|  | AND | R | rs1 AND rs2 -> rd ¦ Bitwise AND |
|  | OR | R | rs1 OR rs2 -> rd ¦ Bitwise OR |
|  | XOR | R | rs1 XOR rs2 -> rd ¦ Bitwise XOR |
|  | SLT | R | rd = 1 if rs1 < rs2 (signed), else 0 |
|  | SLL | R | rs1 << rs2[4:0] -> rd ¦ Logical left shift, zeros inserted |
|  | SRL | R | rs1 >> rs2[4:0] -> rd ¦ Logical right shift, zeros inserted |
| I-type (ALU) | ADDI | I | rs1 + sign_ext(imm[11:0]) -> rd |
|  | ANDI | I | rs1 AND sign_ext(imm) -> rd |
|  | ORI | I | rs1 OR sign_ext(imm) -> rd |
|  | XORI | I | rs1 XOR sign_ext(imm) -> rd |
|  | SLTI | I | rd = 1 if rs1 < sign_ext(imm), else 0 |
|  | SLLI | I | rs1 << imm[4:0] -> rd ¦ imm[11:5] must be 0 |
|  | SRLI | I | rs1 >> imm[4:0] -> rd ¦ imm[11:5] must be 0 |
| Memory | LW | I | Addr = rs1 + sign_ext(imm) -> Mem[Addr] -> rd |
|  | SW | S | Addr = rs1 + sign_ext(imm) ; Mem[Addr] = rs2 |
| Control Flow | BEQ | B | if (rs1 == rs2): PC = PC + sign_ext(imm<<1) ¦ Zero flag -> PCsrc |
|  | BNE | B | if (rs1 != rs2): PC = PC + sign_ext(imm<<1) ¦ ~Zero flag -> PCsrc |